Hardware Implementation of HQC Fixed-Weight Sampling

## Abstract

This project implements a hardware accelerator for the fixed-weight sampling stage used in the HQC cryptographic scheme. The objective is to generate a vector containing a fixed number of unique random positions efficiently in hardware.

Instead of storing and shuffling an entire permutation vector of size N, the design uses a sparse implementation of the Fisher-Yates shuffle algorithm. This significantly reduces memory requirements and improves hardware efficiency.

The design is divided into multiple hardware modules including a random number FIFO buffer, address generation logic, sparse permutation storage, bitset tracking, swap logic, and a finite state machine controller. Together, these modules generate a fixed-weight random vector while minimizing memory usage and computational overhead.

The architecture is designed to be synthesizable, scalable, and suitable for FPGA or ASIC implementation.

## 1. Problem Statement

HQC requires generating a vector containing exactly W unique random indices selected from a large range [0, N−1].

For example:

- N = 17669

- Weight W = 75

A straightforward implementation would create a complete permutation array:

[0,1,2,3,4,5,6,...,17668] and shuffle it. However, storing thousands of values in hardware is expensive. Therefore, this project uses a sparse Fisher-Yates approach that stores only modified positions rather than the entire permutation.

## 2. Overall Architecture

The architecture contains the following modules:

### 1. SHAKE FIFO Buffer

Purpose:

- Receives random numbers from the entropy source

- Stores random words temporarily

- Supplies random values when requested

Input:

- Random 32-bit words

Output:

- Random words for address generation

### 2. Address Generator

Purpose:

Generate a random index: j ∈ [i , N)

This module performs:

range = N − 1 − i

offset = (random × range) >> 32

j = i + offset

This ensures generated indices remain inside the valid range.

### 3. Bitset Tracking Module

Purpose:

Track which positions have already been modified.

Instead of storing all permutation values:

bit=0 → position still contains identity value

bit=1 → position already modified

This avoids storing a huge permutation array.

### 4. Sparse BRAM Storage

Purpose:

Store only modified values.

If a position was never modified:

value(position) = position

Otherwise:

value(position) = BRAM stored value

This drastically reduces memory usage.

### 5. Swap Multiplexer

Purpose:

Perform the Fisher-Yates swap operation.

Inputs:

value_i
value_j

Outputs:

new_i = value_j

new_j = value_i

The sampled output becomes:

sampled value = value_j

### 6. Finite State Machine (FSM)

Controls the complete process.

States:

### IDLE

Wait for start signal.

### INIT

Clear bitset structure.

### SWAP

Perform:

- Generate j

- Read values

- Swap

- Store results

- Output sampled value

### DONE

Signal completion.

## 3. Algorithm

Initialize:

bitset[0..N-1] = 0 , BRAM empty, P[0..WEIGHT-1]

for i = 0 to WEIGHT-1 do

//Generate random index

j = RANDOM(i, N-1)

//Resolve values (identity vs stored state)

if bitset[i] == 0 then

value_i = i

else

value_i = BRAM[i]

if bitset[j] == 0 then

value_j = j

else

value_j = BRAM[j]

// Swap Logic

new_i_value = value_j

new_j_value = value_i

// Update state memory

bitset[i] = 1
    bitset[j] = 1

    BRAM[i] = new_i_value
    BRAM[j] = new_j_value

//Output sample

P[i] = value_j

End for

## 4. Simple Example

Suppose:

N = 10 , WEIGHT = 4.

Bitset: [0 0 0 0 0 0 0 0 0 0]

**ITERATION 0

i = 0

Random gives:

j = 6

- STEP 1: resolve values

i side:

bitset[0] = 0  identity

value_i = 0

j side:

bitset[6] = 0 → identity

value_j = 6

- STEP 2: swap

position 0 gets 6
position 6 gets 0

- STEP 3: store in BRAM

BRAM[0] = 6
BRAM[6] = 0

- STEP 4: update bitset

bitset[0] = 1
bitset[6] = 1

Now:

bitset = [1 0 0 0 0 0 1 0 0 0]

STEP 5: output

P[0] = 6

**ITERATION 1

i = 1

Random gives:

j = 6   ← IMPORTANT (previously used!)

- STEP 1: resolve i

bitset[1] = 0 → value_i = 1

bitset[6] = 1 → NOT identity anymore

So we go to BRAM:

value_j = BRAM[6] = 0

- STEP 3: swap

position 1 gets 0
position 6 gets 1

- STEP 4: update BRAM

BRAM[1] = 0
BRAM[6] = 1   (overwrites old value)

- STEP 5: update bitset

bitset[1] = 1
bitset[6] = 1

Now:

bitset = [1 1 0 0 0 0 1 0 0 0]

- STEP 6: output

P[1] = 0

P so far [6,0,….]

## 5. Why This Approach Is Efficient

A traditional Fisher-Yates implementation requires:

Memory ≈ N values

For HQC128:

N=17669

This becomes expensive.

The proposed architecture stores only modified entries:

Memory ≈ Weight

For HQC128:

Weight=75

Therefore:

- Much lower memory usage

- Better FPGA resource utilization

- Higher scalability

- Suitable for hardware acceleration

Design Sketch